## Input/Output

The Input/Output devices are resources of the computer which the OS must control the programmer's access to.    The way it does this is by not allowing the programmer to access directly the I/O devices.

Rather the programmer asks the operating system to do the input or the output for him.    The operating system is told what type of I/O function is required, the operating system does the function call, and returns any data necessary in a register.    Let us explain it in more detail by use of an example.

### Character Input

There are many types of input that a program may expect (character, string, file, etc.) but for the moment we will look at the simplest one, i.e. inputting a character.    Obviously, we expect that the user will type a character at the keyboard and the operating system will capture that character for us and return it (or rather it's ASCII code) to one of the general purpose registers.    The following program segment will do this :

.model small

.stack 100h

.data

in_char      db	    0		; character variable

.code

mov    ah, 1		; DOS function call 1, character input

int	21h		; Software interrupt 21h Input/output

mov    in_char, al	; character returned in al, stored in variable in_char

mov ah, 4ch

int 21h		; terminate program

end

Points to note:

1.	How does it work? Most of the operating system's I/O services are invoked by using a software interrupt (in particular for MSDOS, it is interrupt 21h).    Using these services, a program can inspect disk directories, make or delete files, read or write records within a file, etc.

The functions available through int 21h are standard and available on any MSDOS system.    The recommended method for using this interrupt is to load the register ah with the function number (and other registers with function specific parameters) and then execute int 21h. When this happens, control is passed back to the operating system temporarily, so that it can execute the particular chosen function call. When the OS is finished, it will return control back to the user program.

2.	What are these function calls for int 21h? There are about 100 different function calls, we are only interested in a small subset for the moment. In particular,

Int 21h - Some of the interrupt’s function calls:

Function Call Number	Description

1	Character Input with echo

2	Character output

8	Character Input without echo

9	Output character string

Thus the first line of the above program indicates which of the many I/O functions that you wish to do, in particular we want to input a character. So the value 1 is placed into the register ah .

After the int 21h is executed, the user program has control again and the ASCII code for inputted character is in the register al.

3.	The behaviour of each of these function calls to int 21h is specific to that function call. So for example to output a string , clearly the string cannot be stored in the register dl (it would not fit in 8 bits), so the behaviour of the int 21h    for outputting strings will clearly have to be different to its behaviour for outputting a single character.

4.	A variable “in_char” is used here. It’s size is defined by db which means data of size byte (we will see later dw and dd, i.e. a data word , 2 bytes and a data double, 4 bytes ).

The variable “in_char” is initialised to 0, and notice that it is declared in the .data segment of the code. We will see more about variables in a future lecture.

Character Output

Again output of a character is a specific MSDOS function call using int 21h, function call 2. However, you also have to provide the character that you wish to output. The ASCII code for this character is put into dl, before you call the software interrupt.

We have seen an example of this in prog1.asm in the previous lecture.

So the following prog2.asm will combine the input of a character from the keyboard and its output to the screen:

; Description: Program to input a character and then output it to the screen

; Registers used:      ax, dx

; Authors: RC/RJ                Class: Cnoc1                    Date: Feb. 1998

title prog2.asm

.model small

.stack 100h

.data

in_char      db	    0	; character variable

.code

mov    ah, 1		; DOS function call 1, character input

int	21h		; Software interrupt 21h, Input/output

mov    in_char, al	; character returned in al,

; is stored in variable "in_char"

mov ah, 02h          	; DOS function call 2, character output

mov dl, in_char        	; the character "in_char" to dl for output

int 21h                  		; print character

finish: mov ax, 4C00h      	; Terminate program

int 21h                  	; correctly

end

This program, when run, will wait for the user to input a character from the keyboard and then cause the character to be output back to the screen.

It is possible to write programs that do not require I/O but we do not want to limit ourselves to this.    We would like to be able to run programs that will work for different inputs.    Mainly we wish to see the output of our programs when they have completed.