## 8086 Assembly Language

Most people learn assembly language to perform programming tasks that are not possible, or easy, with other languages.  Programming in assembly language allows you control every aspect of the computer hardware.  There are many different assembly languages, there is one associated with every microprocessor.  In this course, we are going to learn about programming the 8086 microprocessor.

Assembly language, like high level languages, requires you to enter in the source code of your program.  To do this, typically you will use some kind of text editor.  An assembler, like a compiler for high level languages, translates the source language into object code.  Then many object modules can be combined together to form an executable image that will run on the CPU, called linking.

## Editing a file

You can edit your assembly language program using the Turbo editor.  However that is all you can do in the editor, you must leave the editor to assemble and link your program.  You can call your program any eight letter filename, but use the .ASM extension .

Figure 4.1 Writing an Assembly Language program

## Assembling your program

To assemble your program, we will use Turbo Assembler.  Once you have edited your file and saved it using the .ASM extension, to assemble it, type at the command line:

G>TASM /zi  filename e.g. tasm /zi ex2q6

Figure 4.2 Assembling the program

This will create an .OBJ file on your disk.  When your program has assembled without any errors, you can link the file into an executable program.

## Linking your program

To make a .EXE file from the .OBJ file generated at the last stage, type at the command line:

G>TLINK /v filename    e.g. tlink /v ex2q6

Figure 4.3 Linking the Object Modules to form an Executable

Now you will have a file filename .EXE on the disk. To run this file, type at the command line:

G>filename e.g. ex2q6

## Programs

Programs consist of instructions and data.  Everything in memory is a number, careful organisation is required to prevent the computer interpreting the data as instructions or vice versa.  This is normally left up to the programmer.  However the 8086 microprocessor provides help in organising the instructions and data of your program in memory by using segmented memory.

Memory in the 8086 can be segmented into four regions.  The instructions of your program reside in the CODE segment.  Your data would normally be in the DATA segment.  Two other segments are provided, the STACK and EXTRA segment, the use of these will be explained later.

Figure 4.4 Memory Segments

This shows a possible division of memory.  Each of these segments can be of any length up to 64K.

## CPU Registers

The 8086 microprocessor has a total of fourteen registers that the programmer has access to.

Figure 7.5 CPU Registers

The eight registers, AX, BX, CX, DX, SP, BP, SI, DI are known as general purpose.  Each of the general purpose registers are 16 bit long.  There are four data registers and four index/pointer registers.  Each of the data registers can hold a 16 bit number (i.e. 2 bytes), however the top (high) and bottom (low) byte can be accessed separately. Each half can be used independently, yet it is the same register that is accessed all the time.  The AX register is referenced by AH and AL (H and L refer to high- and low-order bytes).  If you use a data register as an 8 bit register, e.g. AH, then you cannot use its 16 bit parent at the same time, i.e. AX.

The four index registers can be used for arithmetic operations but their real power is used in the addressing modes of the 8086 microprocessor which we look at later.  The two remaining registers are the instruction pointer (IP) and the status word, or flags register.  Neither of these is referenced directly by your program.  The IP, or program counter, is used to store the memory address of the next instruction to be executed.  The status word consists of three control flags and six status flags.  The status flags are used to record specific characteristics of arithmetic and of logical instructions, e.g. the zero flag is set if the result of an arithmetic operation is zero.  The control flags are used to control certain modes of the CPU.

## Representation of data in memory

If we want to store data in memory, there is no problem if the data is just a single byte.  But if it is a word (another name for 2 bytes) then we break that word into its high order and low order byte.  These two bytes are stored in memory in two separate memory locations, the low order byte is stored first.  This means when we are looking at the contents of memory, a word will appear flipped, i.e. the word 4567H would look in memory like 6745H.  This is something we have to be careful of when using the debugger.