# The Stack

As assembly programs get larger, it is often difficult to find places to put data. Recall that the number of  registers is very restricted and having variables in the data segment is not always the best solution since these will take up memory for the duration of the full program.

Often a temporary storage space is required for a small part of the program and can then be released again later. The 8086 CPU has this capability through use of the what is called the stack.

The stack segment can be anything up to 64K (i.e. 65365 bytes) in size.

The stack segment register, ss , holds the segment address of the start of the stack, the stack pointer register, sp, holds the offset from the start of the stack segment to the current position on the stack.

Every program has a stack segment (in the program: .stack), even though they may not use it.  It is a part of memory allocated to a program, that is used in various guises, from parameter passing in procedures to a storage for data.  Later we will see these applications, but for now we simply need to understand what the stack is.

Higher memory

Low memory

pointer to current position, sp

address of start of stack, ss

A stack is just what its name implies, a stack of data elements and can be visualised as follows:

Figure 12.1 The stack data structure

The stack is used normally for temporary storage space . The items on the stack have no names and as with a stack of plates, we add and remove items from the stack at the top. The last item on will be the first item to be taken back off.

For this reason, items are taken back off the stack in the reverse order in which they were put on. We will see this in more detail below.

The data elements that form the 8086 stack are 16-bit quantities.  Any data placed on the stack must be two bytes long.  The sp register is used to contain an offset into the stack, called the stack pointer.  The length of the stack is up to the programmer, as long as it’s total length fits within a 64K physical segment.

Stack Instructions

The simplest instruction that adds something to the stack is push.  push first decrements sp by two bytes, then stores the operand in the word pointed to by sp, (recall that the stack only holds 2 byte  data )

For example, consider the following 2 instructions:

mov	ax, 1234h

push	ax

The result of this can be visualised as:

Higher memory

Low memory

pointer to current position, sp

address of start of stack, ss

12

34

7A

03

Figure 12.2 A push instruction

We can push any variable,  register or  segment register onto the stack provided it is 16 bits long .

However, the following instruction is illegal:

push  al	; can’t push an 8-bit value to the stack.

To retrieve elements from the stack, we use the pop instruction.  pop performs the reverse of push.  The element pointed to by sp is retrieved from the stack and stored in the operand, then sp is incremented by two, for example, the stack in Figure 12.2 above, after the instruction:

pop	bx	; Take the last element off the stack into bx

Higher memory

Low memory

address of start of stack, ss

12

34

7A

03

pointer to current position, sp

Notice that 1234h has not been deleted from the stack, it is now below sp, and is therefore no longer relevant.  The 8086 has no way of knowing if you pop more elements that you pushed, it is thus possible for you to do this, and the data you will get will be garbage.  For this reason, you should always use stack operations in pairs, i.e. for every push, you should have a pop.  Also, you must be careful to pop things in reverse order to the way they were pushed.

As a simple illustration of this idea, consider the following initial register values:

ax = 1234h,	bx = 2A3Ch,  cx = 5672h    and  dx = 5231h

The four diagrams below show the stack after ...

push ax

push bx

push cx

pop dx

Low memory

Higher memory

sp

ss

sp

ss

sp

ss

sp

12

34

ss

2A

3C

12

34

56

72

2A

3C

12

34

2A

3C

12

34

## The value in the dx register will now be 5672h

## The pushf instruction:

## The pushf instruction does not have any operand and it simply pushes the values in the flags register onto the stack.

This is important because the mov instruction will not operate on the flags register. So the only way to copy the flags register into another register is to use the stack as follows:

pushf			; put the flags register value onto the stack

pop	bx		; put this value into the register bx

Notice that pushf does not have an operand.

## Applications

As we said earlier, the stack is used for many applications, one of these is as a versatile temporary storage area.  Often we need registers for a particular operation,  yet they may contain values that we wish to retain.  We saw an example earlier, where their values were stored in a variable temporarily.  This is arduous, since we have to declare a variable for this purpose.  This also has the problem of using this variable twice in a row, thus the second time overwriting the first.  This cannot happen if we use the stack.  Let us look at that example again from lecture 11, now only using the stack, instead of an array:

; The following program inputs a list of 10 numbers, stores them on the stack,

; and then outputs them in reverse order

; Authors: RC/RJ		Class: CNOC1			Date: March 1998

Title prog12.asm

.model small

.stack 100h

.data

command1   db   10,13,'Enter an integer: ','$'

.code

extrn	indec : proc

extrn	outdec : proc

mov	ax, @data

mov	ds, ax			; sets up the data segment register ds

mov	bx, offset command1  ; the start of the string in bx

mov	cx, 10           		; initialise counter to 10

indata: mov 	ah, 9

mov	dx,  bx

int 21h                 		; output command1 string

call	indec           		; Read in an integer

push	ax			; store the inputted number on the stack

loop	indata          		; read in more numbers if cx > 0

mov	cx, 10           		; start counter at 10 again

mov     ah, 2

mov	dl, 13

int 21h                 		; output carriage return

mov     ah, 2

mov	dl, 10

int 21h                 		; output linefeed

next:	pop	ax  			; take next integer from the stack, put into ax

call	outdec          		; output the integer

mov	ah, 2

mov 	dl, ' '

int 21h                 		; output a space character

loop	next            		; go back to output more if cx > 0

mov ah,4ch

int 21h			; terminate program

end