# Procedures

At this stage, you already know the benefit of been able to write procedures.  It takes away the need to repeat certain parts of code in your program. It also helps improve the structure and readability of your code. We have used the procedures “outdec” and “indec” already without discussing procedures in general.

As a simple example, the following is a short procedure, contained as part of a larger program (prog13.asm), to print out a carriage return (CR) and linefeed (LF) every time we want a newline on the screen.

title prog13.asm

.model small

CR 	equ	13

LF  	equ	10

.stack 100h

.data

.code

extrn	outdec : proc

extrn	indec : proc

call	indec		; inputs an unsigned number returns its value in ax

mov	bx, ax		; stores first number in bx

call	newline	; outputs a CR and LF

call	indec		; inputs second number

add	ax, bx		; sums the two numbers, result in AX

call	newline

call	outdec

mov	ah,4cH

int	21H		; terminate program

newline	proc				; procedure called newline begins here

push	ax

push	bx

push	cx

push	dx		; saves the registers on the stack

mov	dl, CR

mov	ah, 2

int	21h		; outputs a CR

mov	dl, LF

mov	ah, 2

int	21h		; outputs a LF

pop	dx

pop	cx

pop	bx

pop	ax		; restores the registers, in reverse order

ret				; return to the caller program

newline	endp				; end of procedure newline

end					; end of full program

Here we have defined a procedure called “newline”, and to invoke the procedure, we call it.  This is nothing new, we have been calling procedures to handle all our numeric I/O (“indec” and “outdec”).  The assembler directives proc and endp mark the beginning and the end of the instructions that make up the procedure.

The ret instruction passes control back to the calling program. One or more ret instructions must appear in the body of the procedure, otherwise execution will not return to the calling program.

## CALL and RET:

The procedure name is defined like any other instruction and constitutes the first instruction of the procedure.  Every time the procedure “newline” is called, the machine executes the instructions of the procedure. When it has finished, it returns to executing the instruction after the call.  How does it do this?

Firstly, the procedure name “newline” can be treated like a label, an thus tells the machine where the address of the first instruction of the procedure is.  But how do we get back to the instruction after the call when the procedure finishes?.  The answer lies with the use of the stack and the ret instruction.

Before passing control over to the procedure, the call instruction places the address of the next instruction immediately after the call onto the stack.  This is the instruction that should be executed after the procedure has finished.  When the ret instruction is executed in the procedure, the return address is poped from the stack and placed into ip.  Thus control passes back to the caller with execution continuing at the proper instruction. This can be viewed as :

2nd

1st

call newline

call outdec

.

.

.

newline proc

.

ret

newline endp

2348h

234Ah

Address

234Ah

stack

sp

pop  ip

now 234Ah

234Ah

stack

sp

3rd

4th

Figure 13.1 The execution of a procedure call

## External Procedures

Procedures do not necessarily have to be in the same code segment as the main program.  In this case, the procedure is declared as before, and in the main program the declaration of the procedure is defined as “external” to this segment, using the instruction extrn.  We have already seen this in our use of the external procedures “indec” and “outdec”.  To define the procedure “newline” as an external procedure, we would have to define a different module as follows:

.model small

CR		equ	13

LF		equ	10

.code

newline	proc			; procedure called newline

public	newline	; makes this procedure accessible to other modules

push	ax

push	bx

push	cx

push	dx		; saves the registers on the stack

mov	dl, CR

mov	ah, 2

int	21h		;outputs a CR

mov	dl, LF

mov	ah, 2

int	21h		; outputs a LF

pop	dx

pop	cx

pop	bx

pop	ax		;restores the registers, in reverse order

ret			; return to the caller program

newline	endp			; end of procedure called newline

end

The procedure is defined as public to all other segments, so that it can be called from another segment. Then in the main module, define newline as external, with extrn:

.model small

.stack 100h

.code

extrn	outdec : proc

extrn	indec : proc

extrn	newline : proc

## Passing Parameters

Parameters or arguments can be passed to procedures in a variety of ways, there is no standard method as with high-level languages.  One simple method is to place the parameters in registers before calling the procedure.  Unfortunately, the number of registers is limited.  Also these registers may have been used in the main program, so we would have to save the register’s values, setup the registers with the parameters and the call the procedure, then after the call, we would have to restore the register’s values.  This practice is also very important, so for all registers that are being used in the procedure, we need to do the following:

pushax

pushbx

push cx

pushdx

callproc1

popdx

popcx

pop  bx

popax

Passing Parameters on the Stack

One of the most common ways of passing parameters to a procedure is to use the stack.  If the parameters are pushed onto the stack before the procedure is called, then the procedure can access them.  How does this work if the return address is also on the stack as well?  Consider the following:

push	param3

push	param2

push	param1

call	proc1

The execution of these instructions would cause the stack to look like:

.

return address

param1

param2

param3

.

sp

We need to access the stack, without modifying sp.  Thus, we cannot use push or pop, since they automatically modify sp.  The bp register is designed for such a purpose. Consider the following, we will access the 1st parameter using bp:

mov	bp,sp	; copy the address in sp into bp

add	bp, 2		; point bp at param1

mov	ax, [bp]	; copy this value into ax

To see this diagramaticly:

.

return address

param1

param2

param3

.

sp = bp

.

return address

param1

param2

param3

.

sp

bp

We can add more to bp to access the other parameters.  When we return, sp still points to the return address, so there should be no problem.  There is one issue left, before calling the procedure, we pushed three parameters onto the stack.  To leave the stack consistent, we need to pop these parameters off the stack.  One way is to pop all three, i.e.:

pop	ax		; param1

pop	ax		; param2

pop	ax		; param3

An alternative is to modify sp directly, i.e.

add	sp, 6	; return sp to position before parameters were pushed.

Finally, a third possibility, with the 8086 we can specify a value to be added to sp on returning to the caller in the ret instruction. e.g.:

ret	6		; return and add 6 to sp