Assembly - Sample questions

Q.1	Using a diagram, explain clearly the relationship between the CPU, memory, input devices, output devices and peripheral devices in a microprocessor.

Give a brief description of each of these parts of a microprocessor with examples where appropriate.

Q.2	There are 3 deliberate errors in the following section of assembly code, find the errors, explain what is wrong and correct the errors:

; Description: Program to print out a new line and then the string 'Hello, World'

; Registers used:   ax, dx

; Segment register used: ds

title prog4.asm

.model	small

.stack	100h

.data

Helomsg	db 'Hello, World',13,10,'$'

CR		db  13

LF		db  10

.code

mov	ds, @data

mov	ah, 2

mov       dx, CR

int  21h

mov	ah, 2

mov       dx, LF

int  21h

mov	ah, 9

mov	dx, offset Helomsg

int	21h

mov	ah, 4ch			;terminate the program

int	21h

end

Q.3	Explain, line by line,  what the following assembly program does and list the output from the program.

.model small

.stack 100h

.data

inchar		db	0

innumber	dw	0

command1   	db   13,10,'Enter a character: ', '$'

command2   	db   13,10,'Enter an integer: ', '$'

command3   	db   13,10,'The character was: ', '$'

command4   	db   13,10,'Double the integer is: ', '$'

.code

extrn	outdec : proc

extrn   indec  : proc

mov	ax, @data

mov	ds, ax

mov	ah, 9

mov	dx, offset command1

int 21h

mov	ah, 1

int	21h

mov	inchar, al

mov	ah, 9

mov	dx, offset command2

int 21h

call	indec           			; procedure to read in an integer into ax

mov	innumber, ax

mov	ah, 9

mov	dx, offset command3

int 21h

mov     dl, inchar

mov     ah,  2

int 21h

mov	ah, 9

mov	dx, offset command4

int 21h

mov 	bx, innumber

add	innumber, bx

mov	ax, innumber

call    outdec				; procedure to output an integer from ax

mov 	ah, 4ch

int 21h

end

Q.4	If the file “prog3.asm” calls the external procedures “indec” and “outdec”, explain clearly how, given the three files “indec.asm” ,  “outdec.asm” and “prog3.asm”, these  can be assembled into one single executable file.

Q.5	Explain, line by line,  what the following assembly program does and list the output from the program.

title ass_Q5.asm

.model small

lowerA	equ	97

lowerZ	equ	122

CR	equ	13

LF	equ	10

.stack 100h

.data

entchar	db	'Enter character: ','$'

.code

mov	ax, @data

mov	ds, ax

again:  mov	dl, CR

mov	ah, 2

int	21H

mov	dl, LF

int	21H

mov	ah, 9

mov	dx, offset entchar

int     21h

mov	ah, 1

int	21H

mov	bl , al

cmp	bl, lowerA

jb	again

cmp	bl, lowerZ

ja	again

outch:	mov	dl, ' '

mov	ah, 2

int	21H

mov	dl, bl

int	21h

inc     bl

cmp     bl, lowerZ+1

je      finish

jmp     outch

finish:	mov 	ah, 4ch

int 	21h

end

Q.6	Explain the meaning of each of the following assembly instructions, giving an example to illustrate each:

(a)	jae,	(b)	jb,	(c)	jne,	(d)	jmp,   	(e)	cmp.

Q.7	The following is a program which attempts to read in 10 integers and then outputs the maximum of the 10.

There are 8 deliberate errors in the program.

Find each of these errors and write out the correct line in each case.

; Example of memory allocation to variables.

; This program reads in 10 integers from the keyboard,

; and outputs the maximum value entered.

title  ass_q7.asm

.model small

TOTAL   equ  10

.stack 100h

.data

innumber   dw	0

command1   db   13,10,'Enter an integer: ', '$'

command2   db   13,10,'The maximum integer was: ', '$'

max	   db	0

.code

extrn	outdec ; proc

extrn   indec  ; proc

mov	ax, @data

mov	ds, ax          	;  Start of the data segment into ds

mov 	cx, TOTAL

again:		mov	ah, 9			;  Prepare to output string

mov	dx, offset command1 ; String to be output

int 21				; Output the string

call	indec           		;  input a number

mov	innumber, ax		;  store it in memory

cmp	cx, TOTAL		; check if is 1st number

jne     not1st ,1

mov     max, ax		;  if 1st number, store as max

not1st:		cmp     al, max		;  compare number to max

jb	notmax

mov	max, ax

notmax: 	loop	again , 3

mov	ah, 9			;  Prepare to output string

mov	dl, offset command2 	; String to be output

int 21h			; Output the string

mov     ax, max      		;  max is to be output

call    outdec			;  output a number

mov 	ah, 4ch

int 21h			; terminate program

end

Q.8	Explain how the “loop” instruction is used in conjunction with the “cx” register in assembly and to what purpose it is used.

Give an example to illustrate your answer.

Q.9	Write an assembly program to output all the uppercase letters of the alphabet in reverse order.

The output will be:

ZYX . . . . . . CBA

( Recall:  ‘A’ = 65 in ascii )

Q.10	Explain what is meant by an interrupt and name at least two different types of interrupt.

What type of interrupt is the assembly instruction “int 21h” ?  Explain in detail what happens when this particular interrupt is called.

Q.11	Write a brief note on any four of the following assembly instructions/directives, giving an example to illustrate the use of each:

(a)	sub	(b)	dec	(c)	jcxz	(d)	loop	(e)	extrn	(f)	equ

Q.12	Write an assembly program which will read in an integer (using the external procedure “indec”), and will only accept an integer between 1 and 26.

Whatever number is entered, the program will then output that number of lowercase letters of the alphabet starting at ‘a’ and going through the alphabet until the correct number is reached.

For example: if 10 is entered, the output will be:

abcdefghij

Q.13	Explain, using diagrams to illustrate your answer, how memory is segmented in a computer.

Explain clearly how the CPU handles the operating system, any other program and data and how control is passed from one to the other.

Q.14	The diagram below shows the 14 different registers in the 8086 processor:

Explain the differences in use of the general purpose registers and the segment registers, using examples to illustrate your answer.