8086 Assembly Exercise 1 (REVISED)

Q1. 	Write a program to initialise register ax with 65535 and bx with 2. Add ax to bx and store the result in register ax. Write down the result you get.

Q2. 	Write a program to initialise the upper half of register ax to 255 and the lower half of dx to 2. Add both numbers. Store the result in register ax. Write down the result you get.

Q.3	Write a program which asks the user to enter a character.

After the character is entered, the program will output to the screen the next character on the ascii table.

Q.4	Write a program which asks the user to enter a letter of the alphabet.

After the letter is entered, the program will output to the screen all the letters of the alphabet which follow after that one, up to and including the letter ‘z’.

Q5. 	Write a program to count from 20 down to 0 in steps of -1.

If the number is > 14 and <= 20 print the message “hot” to the screen.

If the number is > 7 and <= 14 print the message “warm”,

otherwise print the message “cold”.

The following instructions will help with Q.5

mov	cx, 20			; Initialise cx to 20

more:		cmp	cx, 14			; CoMPare cx to 14

jae	big			; Jump to label BIG if result is >= 14

jmp	small			; Else  JuMP  to label small

big:		<code to print message 1>	; print message 1

jmp	next			; now jump to label next

small:		<code to print message 2>	; print message 2

next:		loop	more			; decrement cx by 1(see below) and

; jump back to label more

The loop instruction decrements register cx ONLY by 1 on each iteration. It then jumps to the specified label, in this case label more. When cx reaches 0, execution proceeds on to the instruction immediately after the loop instruction.

The following are the variations on the jump instruction when used with unsigned integers:

jmp	Jump unconditionally

<	jb	Jump on below

<=	jbe	Jump on below or equal

=	je	Jump on equal

>=	jae	Jump on above or equal

>	ja	Jump on above

Not =	jne	Jump on not equal