# 8086 Microprocessor

## Introduction

Figure 1.1 Organisation of a simple microcomputer system

A microprocessor is an integrated circuit that is the basic building block of a microcomputer.  It’s function is to execute the sequence of instructions that make up a computer program.  The program to be executed is stored in the computer’s memory, a separate component to the processor, as can be seen above.  The 8086 microprocessor was developed by a company called Intel (now in Leixlip, Co. Dublin), and was adopted by IBM as the microprocessor for there new computer, The Personal Computer (PC), developed in the late 70’s.  Intel have since developed further microprocessors to replace the 8086, i.e. 80286, 80386, 80486, Pentium, but all are still backwardly compatible with the 8086.

The CPU is the microprocessor chip itself.  Connected to the CPU is memory.  The connection between the CPU and memory is via the bus system (address bus, data bus & control bus).  The microprocessor we will look at is the 8086 microprocessor.  The 8086 microprocessor transfers two bytes (16 bits) to and from memory at a time via the data bus.  This is why it is referred to a 16-bit machine.  A 16-bit machine thus refers to the capacity of the data bus, so a 32-bit machine would have a 32-bit data bus, and would be able to transfer 4 bytes of information between the CPU and memory at any time.

The 8086 microprocessor is designed to manipulate numeric information.  Data can be moved from one place to another in the computer's memory, or data can be moved from memory to registers contained in the microprocessor chip itself.  A register is a temporary storage location internal to the CPU used to store and manipulate data.  They are like memory locations except that the operate faster and serve special purposes for the CPU.  The size of the registers usually reflects the size of the data bus, so that a single register can hold a value which has been transferred from memory.