## Memory

Memory consists of a number of cells.  Each cell can store an 8 bit number, or a byte.  So that we can tell one memory cell from another, memory cells are numbered, starting at zero, up to some maximum size.  In the 8086 architecture, the maximum size of memory is 1Mb (1024*1024 bytes).

The reason for this limit we will see later.

Figure 2.1 Memory Organisation of an 8086 machine

To start with, some definitions are in order.  Memory addresses are given in hex notation.  The memory sizes are referred to in Kilobytes (1,024 decimal), Megabytes (1,048,576 decimal), and Gigabytes (1,073,741,824 decimal).

### Base Memory

Memory available to DOS and programs from location 0 to 640K-1 (9FFFF hex ).  All programs have access to this area.

### Upper Memory

The area between 640K (A0000) and 1M-1 (FFFFF).  This is the area where Upper Memory Blocks (UMBs) are created for loading device drivers, programs, etc. 'high' with DOS.

### High Memory Area (HMA)

The HMA is memory from 1Mb (100000) to 1Mb+64Kb-16 (10FFF0 ), i.e. the first 64Kb-16 bytes of extended memory.  It can be only be used on 286 and up CPU’s.  The HMA is managed as a total unit, i.e. only one program 'owns' it, and 'owns' it all.  This is where most of DOS is placed when you use HIMEM.SYS and the 'DOS=HIGH' command, we will see this in lecture 4.

### Extended Memory

Memory starting at 1M (100000) that is accessible by the CPU.  On a 286, this range extends up to 16Mb-1 (FFFFFF), and on 386 and above CPU’s up to 16Mb-1 (FFFFFF) in 16-bit machine and up to 4Gb-1 (FFFFFFFF) in 32-bit machine:

## Representation of Information in Memory

If we want to store data in memory, there is no problem if the data is just a single byte. But if it is a word then we break the word into its high order and low order byte.  The high order byte is stored first.

## A Computer Program

The physical components of a PC, memory, hard disk, processor, etc., are not enough to make the computer function.  One more ingredient is needed, programs.  When not been run, a program is stored on disk (hard disk, or a floppy, or a CD-ROM).  A program is simply a series of instructions.  When you want to use a particular program, it is copied from disk into memory so that the processor can begin executing those instructions in sequence.

## The Operating System

The operating system can be viewed as a governing body for a computer.  The main resources of a computer system are the hardware, the software and the data.  The operating system provides a means for the proper use of these resources in the operation of the computer system. It's main function is to provide an environment within which the user and the application programs can do some useful work.  We will see how it does this later.

For a program to execute, the program must reside in the memory of the computer  One of the main functions of the Operating System is when a user wants to execute a program, the operating system must be able to load that executable into memory and allow the machine to execute it.  To do this it must be able to find the executable code, load it from the disk into the memory, know where it has put the code in memory and more importantly know the address of the first instruction in memory, and finally, so that the user program can execute, load the program counter with the address if the first instruction.

## What does a computer program look like?

A computer program is made up of a series of instructions which the processor must execute in some sequence.  When you want to execute a program, you usually type in the name of that program at the DOS prompt and the program executes.  As we said earlier, the operating system then loads this sequence of instructions into memory.  This sequence of instructions we refer to as the program’s code segment.  A program needs more than just the instructions, it needs memory space for the data that it is going to use during execution.  The operating system allocates memory space for this data in memory, called the data segment.

At the prompt, you type in the name.  When the prompt is visible, the operating system is executing on the machine.  It is in a infinite loop, waiting for the user to type in a command.  At that time the only thing in memory is the operating system.

Figure 2.2 Memory just after the machine has been switched on

When the user types a command, the operating system will try to execute that command.

G>progname <ret>

The operating system then tries to find an executable file of that name. it looks for the file on the disk. it checks the current directory for the file, if it doesn't find it there, it then checks ail the other directories that are on the search path (PATH in MSDOS).  If it still can find the file, it returns to the user with a "bad command or filename" response.

If it does find the file, it then takes a copy of the it, loads it into memory, across the data bus, and remembers the address of where it puts the first instruction of the machine code.  The operating system dedicated this section of memory for the code of the program.  It also sets up another section in memory for the data that the program may wish to use.  This part is where the program creates the storage required for any of the variables in may want to use.  Now memory looks like;

Figure 2.3 Memory, as a program is running

Finally, the operating system hands over the control to the program to execute.  The machine is now executing the user program and the operating system no longer has control.  The program will continue to execute until it terminates, at which point control will return back to the operating system, and the operating system will go back into it's infinite loop waiting for the next user command.