# Peripheral Devices

Every PC comes with its own standard devices, e.g. screen, keyboard, disk, serial port, etc.  However, there are many different types of device that you may wish to buy for your PC to enhance it usefulness and/or efficiency, like a modem (communications), scanner (input), sound card (output), tape streamer (backup), etc.  All of these devices are not standard to the PC, but still have to be incorporates into the overall PC architecture.  How is this possible?

Every PC comes standard with a fixed number of bays where these new devices can be put, these bays are called expansion slots.

Figure 5.1  Expansion Slots

Into these slots the new device is placed, by putting the device, or a device card supplied with the device (e.g. a scanner) the device has now access to the bus system and thus can communicate with all other devices connected to the buses, in particular, the CPU.

In order for the PC to recognise an external device and be able to communicate with it, the PC needs an extra piece of software to facilitate this communication, called a device driver.  Also, the CPU needs to know when a device has something to communicate to the CPU or memory.  This is important if the device wants to provide data in response to a CPU request or data that was unsolicited.  There a number of ways for the CPU to do this:

- wait until the operation is completed;

- to periodically check the devices in turn to see if they have data;

- to get the device to indicate to the CPU when it has data available.

Historically, the first method was the only way that this could be done.  It meant that the CPU was idle until the device had completed the request.  So for a disk read, the CPU would be idle until the disk had located the data and retrieved it to memory.  The second method is called polling, and is time consuming as the CPU has to periodically stop what it is doing and “ask” all the devices if they have data.  This is very time wasting when the devices have nothing to send.  There are times when the above two methods are appropriate for certain applications, but for now we will leave them.

## Interrupts

The third method is called interrupting.  The device, when it has data to send on the data bus, sends the CPU an interrupt.  Interrupts can be generated by hardware and software.  Think of an interrupt as a doorbell.  If a doorbell wasn’t on your front door, you would have to go to the window periodically and check to see if someone was there, i.e. polling.  Going to the door when the doorbell rings is much more efficient.  An interrupt works like a doorbell for your computer.  An interrupt is a signal to the CPU that an event needing special attention has happened.  When a program is running and such an event happens in some external device, then an interrupt is generated.  When such an interrupt occurs, the system stops the program temporarily and processes the interrupt.  When it has finished processing the interrupt, the CPU returns to running the program.  There are actually many more interrupts generated than just those from new devices.  Some examples of interrupts are the timer interrupt, which is like a metronome for the computer, in that it occurs about 20 times a second, generated by the clock.  Also there is a keyboard interrupt, which is generated every time you press a key.

In the 8086 processor there are four basic types of interrupts:

- Internal Interrupts

- Non-Maskable Interrupts

- Hardware (Maskable) Interrupts

- Software Interrupts

### Internal Interrupts

The 8086 microprocessor generates interrupts that are sensed directly by the CPU, e.g. a divide-by-zero interrupt.

### Non-Maskable Interrupts

This type of interrupt causes the CPU to deal immediately with some kind of catastrophic system failure, e.g. a memory-parity error.

### Hardware (Maskable) Interrupts

These are the interrupts generated by the external devices, as described above.  They are called maskable because you can tell the CPU whether or not to pay any attention to a particular interrupt.  These type of interrupts are handled by the Programmable Interrupt Controller, and can be masked individually.

### Software Interrupts

These are generated by programs, not by hardware.  All DOS and BIOS functions are accessed through software interrupts.  We will see later how to access a software interrupt.