## An Assembly Language Instruction

An assembler processes each line of source program one at a time.  The format of a source line of 8086 assembly language is divided into four fields;

[label] operation [operand] [;comment]

The [ ] indicate that that field is optional.

The easiest way to understand this is to look at an actual program:

; Description: Program to print out the character 'A'

; Registers used:   ax, and dl

; Authors: RC/RJ        Class: Cnoc1          Date: Feb. 1998

title prog1.asm

.model small

.stack 100h

.data

.code

mov ah, 02h      ; DOS function call 2 in register ax (Upper half) means 			 	       ; character output

mov dl, 041h     ; the character to be output 041h (i.e.'A'), is put in register dl

int 21h              ; print the character ( since 2 is in ah) stored in register dl

finish:   mov ax, 4C00h  ; Terminate program

int 21h 	      ; correctly

end

Points to note:

1.	Any characters following the semi-colon ; are comments and ignored by the compiler.

2.	The data and  stack sections will be used in later programs, for reserving memory for variables, fixed constants  etc.

3.	The code section is where the actual assembly instructions are stored.

4.	The mov instruction is used to move data from one place to another.

5.	The instruction int 21h is a DOS software interrupt routine which allows output to the video screen.	If  the value 2 is in the register ah , then int 21h will output to the screen the character stored in register dl.

6.	The finish: above is a label, and the following two instructions will terminate the program correctly and hand back control to DOS. These two lines are critical or the system might hang.

All programs consist of character sequences that look like English words separated by punctuation.  The character sequences are known as identifiers.

### Rules for identifiers:

1. 	The first character must be alphabetic {A..Z,a..z}

2.	Any additional characters can be alphabetic or one of the digits {0..9}

3.	Only the first 31 characters are significant.

### The Operation field

The operation field contains either a machine instruction (e.g. MOV) or an assembler directive (e.g. EXTRN).  Each machine instruction has a special symbol, or mneumonic, associated with it.  Assembler directives have symbolic names that are different from machine instructions, so the assembler is able to differentiate between the two.

### The Operand field

Each machine instruction and assembler directive requires one or more operands.  Operands can consist of constants, variables, special symbols or expressions made up the previous three.

e.g. 	number1+5    is allowed

If more than one operand is required, the operands are separated by a comma;

e.g. 	ADD BX , number1

results in the value in variable number1 being added to the value in BX and the result placed in register BX, the value of number1 is unchanged.

e.g.      MOV  number1, AX + 5

will take the value in AX add 5 to it and the result is stored in variable number1. The value in register AX is unchanged.

### The Comment field

The comment field is used to provide information for the programmer and for others who may examine the program.  A comment should be used on every line of the program, a semicolon must precede the comment.

## 8086 Assembly Language Instructions

The machine instructions of the 8086 microprocessor are grouped together according to the function that they provide.  There are instructions to move data between memory and registers; another group to perform standard arithmetic operations like addition, subtraction, etc.; another group perform only control functions, such as looping.

### Data Movement

Moving data between registers and registers to memory is a basic requirement for any assembly language.  The 8086 microprocessor provides one instruction mneumonic, MOV, for all types of movement and the assembler decides which machine instruction is needed by the operands used with the MOV instruction.

Format of the MOV instruction is:

[label] MOV destination,source [;comment]

The MOV instruction takes the value of the source operand and places a copy of it in the destination operand.  The destination operand can be a register or a variable, but not a constant.  The source operand can be a register, variable or a constant.  Note the direction of the data flow, from right to left.

e.g.  before the instruction if :

AX is 1234H  and   BX  is 5566H

After the following:	MOV AX , BX

then

AX will be 5566H, and BX will be 5566H

The following are not legal MOV instructions:

MOV VAR1,VAR2

We cannot move one memory location to another memory location in one MOV instruction.  Also one must be careful that the size of the source operand in bytes is the same as the size of the destination in bytes.  The following are also illegal  MOV instructions, why?

e.g.	MOV AH, BX

MOV DL, Number1	; if Number1 is a 16 bit variable

also,	MOV 100, AX

Often we need to exchange the contents of two registers, for example AX  and BX, if we try

MOV AX, BX

MOV BX, AX

this does not accomplish the desired result, since the contents of AX are destroyed by the first MOV instruction.

To achieve this

MOV CX , AX

MOV AX , BX

MOV BX , CX

This works, but we required the use of another register CX.  The 8086 microprocessor provides the XCHG instruction.  We can swap between registers, between a register and memory but not between two memory variables. If a swap is performed between a register and memory, the memory variable must be the destination variable.

XCHG AX , BX

XCHG Mydata , DX

where Mydata is a memory variable.

### Addition and Subtraction

The general form of both the add and subtract instructions are:

[label] ADD destination,source [;comment]

[label] SUB destination,source [;comment]

The ADD instruction adds the source and destination operand whether they are words or bytes and places the result in the destination operand.  Similarly SUB subtracts the source operand from the destination operand and the result is placed in the destination operand.  The following are valid:

ADD	AL , AH

SUB	BX , 12      ; 12 here is base 10

ADD	Number1 , 1234h

As with MOV, the size of the source and destination operand must be the same.

e.g.

ADD	AL,  BX

would result in an assembler error.

Also

ADD	AL , Number1

would result in an error also, if Number1 is a 16 bit variable since AL is a 8 bit register.

ADD	AL , AL

would cause the value in AL to be doubled.

SUB	BX , BX

causes the BX register to be cleared. Similarly this can be achieved using:

MOV	BX , 0000H

The Label field:

If a label is present, it is used to associate the symbol with a value.

For example   finish: This is a true label

The value may represent the variable's location in memory, a constant or the location of the instruction in the operation field.  Variables represent data stored in a particular location on memory.  Associated with each variable are three attributes:

1.	The segment containing the variable

2.	offset within segment where the variable is located

3.	type of variable

The type tell how many bytes are needed to store the data.

A symbol in the label field can be made to represent a numeric constant.  Anywhere the symbol appears in the program, it is interpreted as if you had written the constant itself.

A symbol in the label field can also be used to specify a memory location of an instruction.  This is a true label.  These symbols must be delimited by a colon (:).