# Flag Register and Conditional Instructions

There is a special register that the 8086 microprocessor uses called the flag register.  It is not treated in the same way as the other CPU registers.

The contents of this register is not read as a single numeric quantity, rather the values of the individual bits in the flag register are used.

Each of the bits have a specific meaning when they are set (binary value 1) or cleared (binary value 0).  The flag register is a 16 bit register, however not all of the bits are used, in fact only nine are required, as is shown in the figure 10.1

15

0

OF   DF   IF   TF   SF   ZF            AF           PF   CF

Figure 10.1  Flag Register

By far the most important flags are the Carry Flag ( CF) and the Zero Flag (ZF) and these are the ones we will concentrate on.

Occasionally the Sign Flag (SF), the Overflow Flag (OF) and the Direction Flag (DF) are used.

Below is a brief description of each :

OF  -	Overflow Flag	 this is set when the result of an operation becomes too large to fit in the operand it originally occupied.

DF  -  Direction Flag is set by the programmer to tell the CPU whether to read strings in memory from high to low memory addresses or from low to high. By default this flag is zero and memory is read from low addresses to high addresses.

To change the direction, the command std will set the direction flag to 1 and cld will clear the direction flag to zero.

IF  -	Interrupt enable Flag, when IF is set (i.e.=1) interrupts are enabled and may occur when called. When IF is cleared (i.e. = 0) the CPU ignores interrupts.

The instructions sti (Set Interrupt flag) and cli (Clear Interrupt flag) will allow the programmer change this flag setting.

TF  -	Trap Flag is not used very often, it allows DEBUG to execute only one instruction before the CPU calls an interrupt routine.

SF  -	Sign Flag is set when the result of an arithmetic operation forces the operand to become negative (i.e. the highest order bit becomes a 1). Any operation that leaves the sign positive (i.e. the highest order bit is 0) will clear SF to 0.

ZF  -	Zero Flag is set when the result of an operation is zero. If the operand is any non-zero value then ZF is 0.

AF  -	Auxiliary carry Flag is used only with binary coded decimal (BCD) arithmetic.

PF  -	Parity Flag is used to check if the number of set bits in a byte is even or odd. It is used in serial communications through the serial port to detect any errors.

CF  -	Carry Flag is the most useful of all the flags and is used to check if the result of an arithmetic operation or a shift operation causes a bit to be “carried over”.

If a bit is carried over then CF is set to 1, if there is no carry over then CF is 0.

After the execution of an 8086 instruction, some of the flags may be affected.  The majority of flag usage, as seen above, is arithmetic related.

## Now to look in detail at the flags which are used most often i.e. CF , ZF and SF :

## The Carry Flag  (CF) :

If the result of an arithmetic operation exceeds the size of the destination register, then the carry flag (CF) will be set (i.e. put equal to 1).

If it fits in the destination register then there is no carry over and the carry flag would be reset (i.e. put equal to zero).

For example, after the following two lines of code the carry flag will be 1:

or

mov ax, 65535

add   ax, 1

mov al, 255

add   al, 1

Whereas, after the next two the carry flag is zero

mov ax, 200

add   ax, 10

The 8086 provides a group of instructions that allow the testing of each of the flags individually.  These instructions all have the form of a conditional jump, i.e.  the jump is  taken when a certain condition is true.

The condition corresponds to the value or values of one or more of the flags.  If the jump is not taken, then the next instruction in sequence is executed.

The jump on carry instruction (jc) provides a conditional jump based on the setting of the carry flag.  If the carry flag is set to binary 1, then the jump is taken.

e.g.

mov	ax, num1

add	ax, num2

jc	exceed	; jump if  carry flag (cf) is 1.

; result is OK

exceed:				; result exceeds register size

8086 provides complementary instructions for all of its conditional jumps, if we wish to jump when the condition is not true.

The jnc (jump on no carry) instruction is the complement of the jc instruction.

## The Zero and Sign Flags (ZF and SF)

A programmer may want to know when a result is positive, negative or zero.  The zero flag (ZF) and the sign flag (SF) are provided for this purpose.  The zero flag is set equal to 1 if the numeric result of an operation is a zero.

The jump on zero (jz) or the jump on not zero (jnz) instructions will unconditionally jump depending on the value of the zero flag.

For example the following two sets of code are the same, one uses the loop instruction, as seen in the previous lecture, the second uses the jnz instruction mentioned above:

mov cx, 20

again:  call  doThings

loop again

.

.

.

mov cx, 20

again:  call  doThings

dec cx

jnz again

.

.

Points to note:

1.	After the line dec cx  in the above code, the zero flag is 1 when cx reduces to 0, if cx is not zero the control goes back to the label “again”.

2.	In both of these loops the proceedure “doThings” is carried out 20 times exactly.

We can also determine if the result of an operation is positive or negative.  The sign bit is set to binary 1 if the result is negative.

The jump on sign (js) and jump on not sign (jns) instructions test the sign bit.

For example, the two sections of code above could also have been written as:

Goes back to label “again”, as long as the sign is positive.

mov cx, 19

again:  call  doThings

dec cx

jns again

.

.

## Comparisons

We have seen how to compare two values and test the result of the comparison using the cmp instruction.  If two values are equal, then the zero flag is set.  The carry flag, sign flag etc. are used to determine other inequalities.  There are different instructions for testing signed and unsigned numbers, we will use only those for unsigned numbers.

The following table shows the connection between the flag register values ZF, SF, CF and OF and the previous jump instructions which we have used.

For completeness, this table also gives the jump commands for signed numbers

| Meaning | C symbol | Unsigned | Jumps when | Signed | Jumps when |
| --- | --- | --- | --- | --- | --- |
| equal | = | je | ZF=1 | je | ZF=1 |
| not equal | != | jne | ZF=0 | jne | ZF=0 |
| greater than | > | ja | CF=0 and ZF=0 | jg | ZF=0 or SF=OF |
| less than | < | jb | CF=1 | jl | SF != OF |
| greater than or equal to | >= | jae | CF=0 | jge | SF=OF |
| less than or equal to | <= | jbe | CF=1 or ZF=1 | jle | ZF=1 and SF != OF |

; Description: Program to print out the 10 characters starting from 'A'

; Registers used:   ax, cx, dl

; An example of the loop

; Authors: RC/RJ        Class: Cnoc1          Date: Mar. 1998

title prog9.asm

.model small

.stack 100h

.data		count dw 15           	; variable for start of count

endcount  dw  5       	; variable for end of count

.code		mov ax, @data         	; address of data sement into ax

mov ds, ax            	; ds now has address of data segment

mov  bx, endcount     	; endcount into bx

mov ah, 02h     	; DOS function call 2, character output

mov dl, 041h    	; the character 041h (i.e.'A')

again:		int 21h         		; print character

inc dl          		; the next character into dl

dec count       		; reduce counter by 1

cmp count, bx   	; compare counter to end of count

jnz again       		; jump if not zero, i.e. if not equal

finish:         	mov ax, 4C00h   	; Terminate program

int 21h         		; correctly

end